# Setting up PPPoE support

The Cable/DSL router with Point to Point Protocol over Ethernet (PPPoE) authentication is now done through the router.  The login process on the computer is no long used, so they need to be removed. The instruction is here to help assist you in configuring the router.  Before continuing, please make sure you have TCP/IP protocol installed per instructions in the Appendix of the User Guide and your network card is functioning properly.  The documentation assumes that you are using default configuration.  You can reset the router to factory default by pressing the Reset button for 3 to 4 seconds or until the RED Diag LED turns on.

- Go to the web configuration screen for the Cable/DSL router by entering the default IP address 192.168.1.1.

- As shown above, click Enabled for the PPPoE option and enter the User Name and Password as provided by your Internet Service Provider (ISP).

- Click Apply and then click Continue.

- Click on the tab Status and scroll down until you see as shown on the next page.

- Scroll down until you get to the PPPoE section and make sure the PPPoE is Enabled and the status is Connect.

You are done.