# Firmware Upgrade Instructions

The following instructions assume that you are using Windows 98, Me, 2000, or XP.  These instructions also assume that you have Winzip installed.  Otherwise, first download and install the software at http://www.winzip.com.  Please read the entire document before proceeding.

- Download the firmware from the Linksys website and save the file in the My Documents folder.

- Open the My Documents folder and search for the file you just downloaded. The file name may be different then in the example shown below.

- Make sure you have winzip software installed to extract the file just downloaded.  Double-click the file and extract the file to C:\ROUTER as shown below.

- Open My Computer and Double-click Local Disk (c:).  Look for the folder Router that was just created in the previous step.

- Double-click on the Router folder. The files and filenames may be different then those in the example shown below.

Note: The above screen may look different depending on Operating System used and firmware version downloaded.

- Double-click on the Tftp file and a window will appear as shown below.

Note: The above screen may look different depending on the Operating System used and the TFTP program version.

Fill in the following:

Server: 	Enter the IP address of the router.  The default IP address is 192.168.1.1

Password:	Enter the password for the router.  The default password is admin (the password will not be displayed)

File:	Enter the location of the .bin file.  The filename may be different from the example below, depending on what firmware version you have downloaded.  To browse for the file, click the “…” button next to the File prompt and find the C:\Router directory. Double-click on the .bin file and click the Open button to accept the file.

Note: The filename maybe different depending on the version of the firmware you have downloaded. To display the file type, Click on the file and point your mouse on the filename, and a yellow pop-up message will appear.

- After you click the Open button, you will be back in the Upgrade Firmware window.  Click the Upgrade button to start the upgrade.

- A window will appear showing you the progress. Once the firmware upgrade has been successfully completed, you can close any open windows.

Firmware upgrade is complete.