Changing the Product ID on the Novatel U740 Data card.

Identifying the Data Card. This is an image of the unbranded Vodafone Mobile Connect Novatel U740 Data Card. As you can see there is no branding anywhere on the front of the card.

Right Click on My Computer and Click on Properties

Click on the Hardware Tab and click on Device Manager.

In Device Manager Expand the Modems Item and right click on the Novatel U740 Modem and select Properties.

Click on the Details Tab and Check the PID_1400 item. If it  displays PID_1400 it is the correct Item, if it displays PID_1410 Use the Utility  attached at the bottom to Change the PID.

Identifying the product ID.

Unzip the utility into a directory and run the exe file.

Insert the Card and Click on VF PID. It takes about 20 Seconds. Once this has completed  remove the data card and re-insert. It may be necessary to re-create the profile in the dashboard.