### Quick Start

If you have ever used Grand IP Video Sever, please to delete

“ KENCAPI Class ” first

C:\Windows\Downloaded Program Files\KENCAPI Class

Step 1：Please connect power adapter to Grand IP Video Server 4 Port first.

Step 2：Connect to video / audio connector of video / audio source.

( Camera / DVD player / mp3 / microphone)

Step 3：Connect to video / audio input of video / audio source (monitor / DVR) for

monitoring picture and sound in the local end.

Step 4：Plug RJ-45 Cable into your router, likes the following figure.

Step 5：Execute the “ipEdit.exe” in the CD.“ and click “Update”.

http://192.168.1.127/” has been listed in the left side, and double click        http://192.168.1.127/” to login the IP Video Server.

PS. Please refer to the manual in the CD ROM.