IP CAMERA

User’s Manual

About this document

This user manual is intended for administrator and/or user of IP camera. Operational instructions & network configuration are elaborated. Prior knowledge about network setup and configuration will be helpful.

IMPORTANT

- Check PC specification requirements.

- Check OS platform requirements.

- Install NVR2 software before you start working with video server.

- Read special notes and important configuration information.

TABLE OF CONTENTS

# INTRODUCTION

## 1.1	PRODUCT OVERVIEW

CMI-220 is a professional IP surveillance application. With embedded web viewer, streaming video/audio over network can be accessed like browsing a website. It is also capable of encoding MPEG4 stream, the only digital surveillance product in the market with proprietary design for "Ease-of-use", "Reliability", and "Manageability".

## 1.2	PACKAGE CONTENTS

IP Camera				       Software CD

Power connector           			Power Adaptor

15

215

1.3	PHYSICAL DESCRIPTION

Back Panel

45

35

5

- Power Socket

DC12V 1A

- Video Out

- Reset Button

- LAN/WAN RJ45 Port

10/100 Mbps auto sensing, fast Ethernet port for secure Local Network or WAN for FTTH or xDSL connectivity

- DI/DO

DO:	Logic Level 0:	0~0.5V;	Logic Level 1: 2.8~3.3V

Side Panel

1. Auto Iris

2. V.D  D.D

Video Driver or Digital Driver

3. VR

Variable Resistant adjustment

4. DIP Switches for CCD Functions

Auto Electronic Shutter Speed Mode

AES On 1/50 (60) sec. ~ 1/100,000 sec;

AES Off: 1/50 (60) sec.

Back Light Compensation

ON/OFF (Switchable)

Flickless

ON/OFF (Switchable)

## 2	Hardware Installation

Make sure network environment

Default IP of IP Camera is 192.168.1.2 Please make sure IP Camera and your PC are in the same network segment before running the installation.

Please set the number from 2 to 254.

e.g. 192.168.1.115

Connect necessary cables

The Ethernet cable and power cable are necessary for previewing function.  Make sure all cables are correctly and firmly connected.

- Connect an analog monitor to IP Camera Video Out  (BNC connector)

- Connect the power adaptor to IP Camera

- Connect IP Camera LAN/WAN port to an Ethernet hub (RJ45 connectors)

- Connect a PC to the Ethernet hub (RJ45 connectors)

# SOFTWARE CD

Software CD in the package contains the following:

- NVR2 IP surveillance management software

- NVR User Manual

- Firmware binary file

- IP Camera Manual

- IP Setting

# BEFORE YOU INSTALL SOFTWARE

- PC running Microsoft Windows 2000 or above is only capable of running NVR2 software.

- PLEASE go through NVR users’ manual before installation to check further PC requirements in order to ensure smooth software operation.

# LANGUAGE SUPPORT

ENGLISH

NOTE

NVR setup will auto-run when inserting CD. Pay attention to dialog box questions before you press OK button.