NVR System

User Manual

Version 1.1

TABLE OF CONTENTS

1.	INTRODUCTION	3

2.	PLATFORM REQUIREMENTS	4

3.	PC REQUIREMENTS	4

4.	INSTALLING NVR	4

5.	SEGMENTED DISPLAY	8

6.	IPS NODE-CHANNEL TREE	12

ADDING A NODE	12

REMOVING A NODE	15

MODIFYING CONFIGURATION OF A NODE	16

7.	VIDEO	16

BITRATE SELECTION	17

VIDEO QUALITY	17

8.	AUDIO	18

9.	PLAYBACK and RECORDING	19

10.	CAMERA PTZ controls	27

# INTRODUCTION

NVR is primarily a surveillance video viewer and multi-purpose management tool for encoder IP surveillance products on LAN or WAN environment.

Multi-purpose management includes…

- Multi-segment display

View multiple video streams simultaneously in segmented screen mode or combinations of various segments (1/4/7/9/10/13/16) with single click. Full screen view is fully supported for each segment combination.

- IPS Node – Channel tree

Tree structured view of active encoder nodes with up to 4 channels under each node. Add/remove and configure individual nodes with dedicated push buttons.

- Video

Event triggered and schedule based video recording. Functions are restricted on guest account. Video resolution and bit-rate control selection is available.

- Audio

Individual channel audio monitoring and recording with scheduling options. Support only audio in at the moment

- Playback and Recording

Video and audio recording with image capture and indicator for disk usage is available.  Event selection for playback with speed control, event time line and calendar for easy selection.

- Camera control

Pan / Tilt / zoom control if supported by camera and add/remove easily by single click.

# PLATFORM REQUIREMENTS

NVR supports the following operating systems.

Microsoft Windows 98SE

Microsoft Windows ME

Microsoft Windows 2000 professional

Microsoft Windows XP Professional / Home

Microsoft Windows Vista Premium

# PC REQUIREMENTS

- IBM-PC compatible computer with Microsoft Windows operating system.

- Display resolution – 800 x 600 high color (16 bit) or higher.

- Free hard disk space – at least 40 Mbytes or higher if used extensively for recording surveillance video.

Proper installation is not guaranteed on PCs which don’t meet the above requirements.

# INSTALLING ETROSTATION

Click open folder with “IPSurveillance_Setup.exe” files.

Folders appear as shown below

Click open “IPSurveillance_Setup.exe” file.  Then click “Install IPSurveillance” icon

System will install FFDShow software decoder first, click to “next” to go ahead

.

After ffdshow installation finised, system will automatically to start to install NVR software, see following display, then click “next”

Click “Next” to proceed or “Cancel” to exit.

Setup has completed. Click “Finish” to exit.

Double check IPSurveillance is located at START->PROGRAMS->IPSurveillance

# SEGMENTED DISPLAY

The main screen is a viewer for surveillance video from various encoder IPS products within a LAN/WAN environment up to a maximum of 16. The screen is segmented in various fashions depending on user’s choice of viewing comfort.

Full screen mode is supported in all combinations of segments available. Segment combinations are (1/4/7/9/10/13/16). Each view has a dedicated button for easy access and turns to red color to indicate selection.

“Full screen” literally means that display extends to the very corner of the monitor. To exit full screen mode, press ESC key on your keyboard.

The partitioned screen of every configuration of segments is shown below.

1 Segment

4 segments

7 segments

9 segments

10 segments

13 segments

16 segments

Full Screen

TIME /DATE/CAMERA name is displayed in each segment.

Time indicates local time on the PC running NVR.

If the video stream is disconnected from the encoder machines, the following message appears on the corresponding segment.

Individual segments can be selected by pressing on the segment number listed on segment view buttons.

# IPS NODE-CHANNEL TREE

To get information about the IPS nodes (EV3000 series based) on the LAN/WAN environment, Press this button once.

If there are no nodes added yet, then the panel looks blank. On top, there is local date/day/time information.

## ADDING A NODE

- Press   button to the add a node.

- A dialog box will appear as shown below.

- Fill in “Node Name” of your choice.

- If you know “IP address”, directly key in.

- Select “port number” if you have specific requirements.

- To enable audio, check “Enable SPK” check box.

- To enable microphone, check “Enable MIC” check box.

- To enable “internal codec”, check the box accordingly.

- “Frames per second” settings may be available depending on node capability.

- Up to a maximum of 50 nodes can be added.

- Check the “guest” check box, if you would like to view with restricted functionality.

- Type “login” information if you are aware of it as shown below.

Click “OK” to apply settings and “Cancel” to exit.

Click “Search” to auto-search available nodes on LAN/WAN.

After 1 or 2 seconds, a dialog box will appear as shown below.

Enter “Node Name” to search by name if you are aware of it.

| If IPS nodes have been already added, the corresponding channel information is displayed under each node. Node names are also displayed for each channel as shown below. | Click “Refresh” to view latest available nodes. Click “OK” to add the node. Click “Cancel” to exit without adding the node. |
| --- | --- |

Each channel is marked by multi-colored icons which represent status of the selected channel as listed below

|  | IP Product |
| --- | --- |
|  | Status of idle |
|  | Monitoring |
|  | Monitoring+Recording |
|  | Motionn Detection |
|  | Digiral IN |
|  | Monitoring+Reserve Recoring |
|  | Unavailable |

## REMOVING A NODE

- Select the IPS node you would wish to remove.

- Press   button to the remove a node (not Channel).

- A confirmation dialog box will appear as shown below.

- Click “Yes” to confirm removing the node.

- Click “Cancel” to exit removing the node.

- Notice that the node has disappeared from the list.

NOTE: Alternatively, you could initiate the node removal procedure by selecting the node, and then making a right click on your mouse and selecting “Delete Server Info”.

## MODIFYING CONFIGURATION OF A NODE

- Select the IPS node you would wish to reconfigure.

- Press      button to the reconfigure a node (not Channel).

- A confirmation dialog box will appear as shown below.

- It is similar to “Add Node Info” dialog box.

- Choose your appropriate settings.

- Click “OK” to save configuration information.

- Click “Cancel” to exit removing the node.

- You can notice the “Reconnecting ..” message on the grayed display segment of the selected node which auto-refreshes the video stream according to your latest configuration . Wait for few seconds to see the video stream restored back to normal.

# VIDEO

Video display panel controls quality and resolution of the transferred image. Resolution control is unavailable for “guest” account. Video panel is different according to the selected model.

Encoder products support the following formats at the corresponding resolution. (The ones in grey are not valid yet)

|  | D1 | 4CIF | VGA | CIF | QVGA |
| --- | --- | --- | --- | --- | --- |
| NTSC | 720X480 | 704X480 | 640x480 | 352x240 | 320x240 |
| PAL | 720X576 | 704X576 | 640x480 | 352x288 | 320x240 |

The video type can be selected with its dedicated push buttons as shown below. Selected button appears red to indicate selection.

## BITRATE SELECTION

Bit rate determines the smoothness and quality of the video displayed for each channel. It can be modified if the IPS product is capable of supporting it. The value ranges from 64Kb ~ 4Mb and can be selected in a drop-down list as shown below.

Bit rates: 128k, 256k, 384k, 500k, 750k, 1M, 1.5M, 2M, 4M (bps).

Users should select a suitable bit rate accordingly.

## VIDEO QUALITY

High image quality can result slow transmission speed because of the limit of the network bandwidth. Therefore, fps of transferred image drops resulting in very poor video quality.

It is recommended to maintain an appropriate balance of bit rate and quality of video based on your specific LAN/WAN conditions.

# AUDIO

Audio Panel works only when the IP Product has audio module.

This is available for the selected channel of the IP Product. For activation of audio, press SPEAKER button. Button color changes to red after click indicating selection. (MIC is not valid yet as there is just audio in)

OFF    ON

Settings used in the “Modify Server Info” dialog box for using audio functions of a IP Product is shown below.

NOTE: It is possible to use the speaker function for a “Guest” account. But MIC cannot be enabled.

# PLAYBACK and RECORDING

Playback and recording of individual channels can be done with manual selection or as a scheduled event.

Select a channel on Camera List or the image on segmented screen and click         button.

Alternatively, you could click “Start Recording” after selecting the channel and clicking the right mouse button.

By clicking “▼” user can see the status of the other disks.

Disk shows the free space available for recording. Recording process starts until “Stop Recording” is clicked on pop up menu of the image on Screen or the channel on Camera List. Schedule recording is a useful function to manage the disk space effectively.

When DIO event occurs recording process proceeds for the time under the line only when the IP Product takes the signal by a sensor or a motion detector.

When motion event occurs, recording process proceeds for the following time under the line only when the IP Product takes the signal by an internal motions detector.

For this function, the motion detection function must be activated.

The recording time by Event function is up to 20secs.

Recorder settings:

The time interval can be specified. And the upper disk limit can be set to avoid over usage of disk space.

Joystick can be connected and its serial port selection and baud rate should be chosen as well. Clicking “connect” will allow the user to handle a joystick input.

Change storage location

The storage location can be changed by clicking the “change storage location” button.

Click “OK” to save. Click “cancel” to exit and click “Browse” to 		choose another location.

Change password

Password can be changed by “Change Password” button and 	needs to be confirmed to save.

Click “OK” to save password.

Click “Cancel” to exit password change.

Play back function

Press the playback button to start playback. Calendar on the right 	makes it effectively easier to pick a date to view recordings on that 	day.

Step 2

Event based UI option allow user to select the recorded video by 	timeline and channel for each IP. Slider on the right lets the play  	back be done quickly in multiples of minutes.

The Channel information is displayed under a list of IP addresses. Up 	to four channel information is shown under each IP.

The recorded files are named as from – to timeline for easy 	identification.

Three combo boxes one for IP, one for channel and one for time 	snaps allow easy selection.

Recording is always indicated in “RED” color as shown below.

Right click provides an easy means to select your options.

Captions allows user to choose specific details such as FPS, Channel,   	Video Size and event.

.

Web view allows user to view the home page using a standard 	browser on a PC.

The WEB UI is shown with all available control listed on the bottom 	including bitrates, Audio, resolution, Pan, Tilt, Zoom, and Focus.

Edit recording schedule by selecting from the standard right mouse 	button click.

Scheduled Recording:

This option allows user to choose the disk to record. Specified node 	information. Allows selection of DIO/Motion detect event and for 	how long.

The start/end time, date can be specified.

Click “OK” to save and “Cancel” to exit.

Extensive recording schedules including each day of the week can 	be recorded as well including timing for each day.

# CAMERA PTZ controls

Cameras connected to encoder products which support PTZ (Pan/Tilt/Zoom) module can be controlled from NVR. Suitable control protocol must be pre-setup. IP camera’s administrator web page will have this information.

“Guest” accounts have no privilege to control PTZ functions but viewing only. The buttons controlling PTZ available on NVR2 are shown below.