HOLUX ezUpdate Operation Procedure

HOLUX ezUpdate V4.00-4 support device

GPSport260

HOLUX ezUpdate V4.00-4 attention

- Do not disconnect cable from device or PC site in process of updating. - Please make a copy and delete the Log data in before update.

Step1：Install USB driver

GPSport260：Install PL2303_Prolific_DriverInstaller_v1210.exe

Step2：Connect USB cable to device

Step3：Execute HOLUX ezUpdate

Execute HOLUX ezUpdate.exe

If detect COM Port，press Connect button.

If fail to detect COM Port，select COM Port and then press Connect button.

[The way to select COM Port，please refer to Q&A1]

Display information (Device Type / Firmware Version)

Step4：Select firmware file

Press Firmware Select button，select firmware file will update (.bin file ends with _HLX).

Display firmware information

(Adapted Device / Firmware Version)

Step5：Update Firmware

Press Execute Update button，it will take about 2~3 minutes to update.

If you update GPSport260，please follow the procedure：

Device begins to download firmware and do not disconnect USB cable.

Update successfully

Step6：Restart device

Please restart device to confirm the setting values will work.

Q&A1：How to set COM Port

Open Device Manager，under Ports item：

GPSport260：

COM Port of “Prolific USB-to-Serial Comm Port”.

And select COM Port in the ezUpdate COM Port list.

Q&A2：COM Port can’t be set

If COM Port can not detect COM Port and COM Select still can not detect COM Port or the cursor in the monitor shift irregularly.

Please follow the operation：

Step1：Turn Off device

Step2：Re-connect USB cable

Step3：Re-execute ezUpdate

Q&A3：Exceeding field

If you execute ezUpdate first time，the fields are exceeding。

Please follow the steps to fix it。

Step1：Press right button of mouse on desktop，select “Properties”

Step2：Setting tab and Advanced button

Step3：General tab and Font Size，select “Small Font”

System will restart and fields will not exceed。