OXFW911 Firmware 3.7

Change Report

27 Jan 2003

# Release Changes

Since version 3.6, the following changes have been made:

ATA-6 master and slave (dual drive) capability is supported when hardware modification is present.

Hardware modification presence is automatically detected.

If no hardware modification found, switch to ATA-5 reporting (128GB max).

If no hardware modification found, but one or two vendor specific (nonstandard) ATA-6 drives are present, use the vendor specific operation to allow ATA-6 capability.

New CSRs introduced, reporting whether hardware modification is present and whether standard or vendor specific ATA-6 operation is used.